Installation Guide of FC Card Driver in VMware System

目录

1.	Precautions for operation	3

2.	Preparation before operation	3

3.	Driver Installation	3

4.	Driver Uninstallation	4

Precautions for operation

- This driver version is only applicable to VMware 7.0.3

- This driver is suitable for FC cards.

- Driver upgrade requires server restart to take effect.

Preparation before operation

- Query the current operating system kernel.

#uname -a

- Query the current driver version. The driver version is 4.1.14.0.

#esxcli software vib list|grep qlnativefc

Driver Installation

- After decompressing the driver package, upload the driver file MVL_bootbank_qlnativefc_5.3.82.01OEM.703.0.0. 18644231.vib to the server operating system，such as the /tmp directory，and execute the driver installation command.

#esxcli software vib install -v /tmp/MVL_bootbank_qlnativefc_5.3.82.0-1OEM.703.0.0.18644231.vib  --no-sig-check

- Reboot the system.

#reboot

- Check if the driver version has been successfully upgraded. As shown, the driver version is 5.3.82.0 now.

#esxcli software vib list|grep qlnativefc

Driver Uninstallation

- Execute uninstallation command

#esxcli software vib remove -n qlnativefc

- Reboot the system.

#reboot

- Check if the driver version has successfully uninstalled.

#esxcli software vib list|grep qlnativefc