Installation Guide of ngbe Driver in Ubuntu System

目录

1.	Precautions for operation	3

2.	Preparation before operation	3

3.	Driver Installation	3

4.	Driver Uninstallation	4

Precautions for operation

- This driver version is only suitable for installation and use under the regular kernels of Ubuntu 14.04-24.04 operating systems.

- Driver version upgrade requires rebooting to take effect.

- The screenshot shown in this document uses Ubuntu 24.04 to upgrade the driver of version 1.2.6.4 to demonstrate the driver upgrade process.

Preparation before operation

- Query the current driver version.

#modinfo ngbe|grep -i version

- Before installing the driver, you need to install the following dependencies. If other dependencies are still missing during the process, install them according to the prompts.

- #sudo apt install unzip make gcc

- Execute the following command to back up the system img file：

#cp -rf /boot/initrd.img-`uname -r` /boot/initrd.img-`uname -r`_back

Driver Installation

- After decompressing the driver package, upload the driver file ngbe-<x.x.x>.zip to the server operating system，such as the /home directory，and execute the driver installation command under root user.

- #cd /home

#tar xvf ngbe-<x.x.x>.zip

#cd ngbe-<x.x.x>/src/

#make install

- Reboot the system.

#reboot

- Check if the driver version has been successfully upgraded

#modinfo ngbe |grep -i version

Driver Uninstallation

- Enter the directory where the source code is located and execute the driver uninstallation command.

- #cd ngbe-<x.x.x>/src/

- #make uninstall

- Reboot the system.

#reboot

- Check if the driver version has successfully rolled back to the inbox version.

# modinfo ngbe|grep -i version