Installation Guide of ngbe Driver in Ubuntu System

目录

1.	Precautions for operation	3

2.	Preparation before operation	3

3.	Driver Installation	3

4.	Driver Uninstallation	4

Precautions for operation

- This driver version is only applicable to Ubuntu 22.04 (5.15.0-25-generic). Before installation, please make sure to confirm that the system version and supported kernel version are consistent before operation.

- Driver version upgrade requires rebooting to take effect.

- The screenshot shown in this document uses Ubuntu 20.04.6 (5.15.0-67-generic) to upgrade the driver of version 1.2.7 to demonstrate the driver upgrade process.

Preparation before operation

- Query the current operating system kernel.

# uname -a

- Query the current driver version.

# modinfo ngbe | grep -i version

- Execute the following command to back up the system img file：

# sudo cp -rf /boot/initrd.img-`uname -r` /boot/initrd.img-`uname -r`-bak

Driver Installation

- After decompressing the driver package, upload the driver file ngbe-<x.x.x.x>.deb to the server operating system，such as the /home/zteroot/ngbe_driver directory，and execute the driver installation command under root user.

# cd /home/zteroot/ngbe_driver

# dpkg -i ngbe-<x.x.x.x>.deb

# update-initramfs -u

- Reboot the system.

# reboot

- Check if the driver version has been successfully upgraded

# modinfo ngbe |grep -i version

Driver Uninstallation

- Execute uninstallation command

# dpkg -l|grep ngbe

# dpkg -r + driver package name

- Reboot the system.

# reboot

- Check if the driver version has successfully rolled back to the inbox version.

# modinfo ngbe | grep -i version