Installation Guide of Wangxun ngbe Driver in Linux System

目录

1.	Precautions for operation	3

2.	Preparation before operation	3

3.	Driver Installation	3

4.	Driver Uninstallation	4

Precautions for operation

- This driver version is only applicable to OpenEuler 22.03 LTS SP3 (5.10.0-182.0.0.95.oe2203sp3.x86_64). Before installation, please make sure to confirm that the system version and supported kernel version are consistent before operation.

- Driver version upgrade requires rebooting to take effect.

- The screenshot shown in this document uses CGSL 5.04 （3.10.0-693.21.1.el7.x86_64） to upgrade the driver of version 1.2.6 to demonstrate the driver upgrade process.

Preparation before operation

- Query the current operating system kernel.

#uname -a

- Check if the kernel files have been installed. If kernel-devel、kernel-headers have not been installed, install the files and upgrade the driver. You can configure the yum repository (refer to the Appendix) and proceed with installing each file individually.

#rpm -qa|grep kernel-devel-`uname -r`

#rpm -qa|grep kernel-devel-`uname -r`

- Query the current driver version.

#modinfo ngbe|grep -i version

- Execute the following command to back up the system img file：

#cp -rf /boot/initramfs-`uname -r`.img /boot/initramfs-`uname -r`_back.img

Driver Installation

- After decompressing the driver package, upload the driver file ngbe-<x.x.x>.x86_64.rpm to the server operating system，such as the /home directory，and execute the driver installation command under root user.

#cd /home

#rpm -Uvh ngbe-<x.x.x>.x86_64.rpm

- Reboot the system.

#reboot

- Check if the driver version has been successfully upgraded

#modinfo ngbe |grep -i version

Driver Uninstallation

- Query the name of the installed driver package first.

#rpm -qa|grep ngbe

- Execute uninstallation command

#rpm -e + driver package name

- Reboot the system.

#reboot

- Check if the driver version has successfully rolled back to the inbox version.

# modinfo ngbe|grep -i version

- Appendix

Configure the yum repository:

- Taking redhat8. X and redhat7. X as examples, the steps to configure the yum source are as follows:

- Redhat8. X mounting configuration source file writing：

Using rhel-8.3-x86_64-dvd-Hygon-v8.iso as an example, upload the rhel-8.3-x86_64-dvd-Hygon-v8.iso to the home directory：

#cd /home

#mkdir /mnt/（Create a new mnt directory in the home directory）

#mount /home/rhel-8.3-x86_64-dvd-Hygon-v8.iso /mnt/（Mount the image file of the operating system in the directory of the previous step）

#cd /etc/yum.repos.d/（Enter the yum.repo.d directory）

Move the remaining repo files to another directory first：

#mkdir bak

#mv *.repo bak

Create and edit new file redhat.repo：

#touch redhat.repo

#vi redhat.repo

Press the i key to start editing:

[redhat]

name=redhat

baseurl=file:///mnt/AppStream

enabled=1

gpgcheck=0

[rh1]

name=rh1

baseurl=file:///mnt/BaseOS

enabled=1

gpgcheck=0

Press ESC to exit editing, enter :wq in order to save the file.

#yum install +dependency packages (Try installing in sequence according to error prompts)

- Redhat7. x mounting configuration source file writing：

Taking CentOS-7.6_x64_Hygon.iso as an example，upload the image file to the home directory：

#mkdir /mnt/（Create a new mnt directory in the home directory）

#mount /CentOS-7.6_x64_Hygon.iso /mnt/

Edit repo file：

#cd /etc/yum.repos.d/

Move the remaining repo files to another directory first：

#mkdir bak

#mv *.repo bak

Create and edit new file soft.repo：

#touch soft.repo

#vi soft.repo

Press the i key to start editing:

[soft]

name=soft

baseurl=file:///mnt/

enabled=1

gpgcheck=0

priority=1

gpgkey = file:////etc/pki/rpm-gpg/RPM-GPG-KEY-redhat-release

Press ESC to exit editing, enter :wq in order to save the file.

- Dependency Packages Installation：

#yum install + dependency packages（Try installing according to the error prompt in sequence）

#yum install kernel-devel-`uname -r`

#yum install kernel-headers-`uname -r`