Installation Guide of PMC RAID Card Driver in VMware ESXi System

目录

1. Precautions for operation	3

2. Preparation before operation	3

3. Driver Installation	3

4. Driver Uninstallation	4

Precautions for operation

- This driver version is only applicable to VMware ESXI 8.0 U3/U2.

- This driver is supported for PMC RAID card.

- Driver upgrade requires server restart to take effect.

- The screenshot shown in this document uses VMware ESXI 8.0 U2 to upgrade the driver of version 80.4662.0.112 to demonstrate the driver upgrade process.

Preparation before operation

- Query the current operating system kernel.

#uname -a

- Query the current driver version. The driver version is 80.4495.0.5000.

#esxcli software vib list|grep smartpqi

Driver Installation

- After decompressing the driver package, upload the driver file Microchip-smartpqi_<x.x.x.x>.zip to the server operating system，such as the /tmp directory，and execute the driver installation command.

#esxcli software component apply -d /tmp/Microchip-smartpqi_<x.x.x.x>.zip

- Reboot the system.

#reboot

- Check if the driver version has been successfully upgraded.

#esxcli software vib list|grep smartpqi

Driver Uninstallation

- Execute uninstallation command

#esxcli software vib remove -n smartpqi

- Reboot the system.

#reboot

- Check if the driver version has successfully uninstalled.

#esxcli software vib list|grep smartpqi